Nathan Somevi - Technical Rider

Hybrid guitar / tenor sax / keyboard / drums

Version: draft for advance | Contact: [insert booking/tech email]

Overview

This rider covers a four-piece setup built around hybrid guitar, tenor saxophone, keyboard and drum kit. The hybrid guitar uses a split rig: guitar-side signal to a guitar amp and bass-side signal to a bass amp. The tenor sax setup uses a normal dry mic plus a separate wet/reverb channel where possible.

Assumption: the exact amp ownership and keyboard/backline provision should be confirmed during advance. This rider is written so it works whether the artist brings some gear or the venue provides backline.

# Stage plot

# Preferred stage layout

Hybrid guitarist: stage right, with guitar amp and bass amp behind or slightly behind/side of player.

Tenor sax: front centre or centre-right, with two channels available at the sax position.

Keyboard: stage left, stereo DI preferred.

Drums: upstage centre.

FOH stage box/multicore downstage or centre stage if possible.

# Input list

| Ch. | Source | Mic / DI | Stand / power | Notes |
| --- | --- | --- | --- | --- |
| 1 | Hybrid guitar - guitar amp | Dynamic guitar mic, SM57/equivalent | Short boom | Clean/warm guitar side. Avoid excessive low end. |
| 2 | Hybrid guitar - bass amp | DI from bass amp preferred, or dynamic mic | DI or short boom | Low/bass side of hybrid guitar. Keep full but controlled. |
| 3 | Tenor sax - dry | Clip-on or stand mic | Tall boom | Main sax channel. Natural sound, light/no reverb. |
| 4 | Tenor sax - wet/reverb | Second mic or split of sax mic | Tall boom or channel split | Large plate/hall reverb. Keep separate from dry sax channel. |
| 5 | Keyboard L/Mono | DI | Mains power | Use L/Mono if mono only. |
| 6 | Keyboard R | DI | Mains power | Stereo preferred where practical. |
| 7 | Kick drum | Kick mic | Short boom | Standard kit mic. |
| 8 | Snare top | Dynamic mic | Short boom | Standard kit mic. |
| 9 | Hi-hat | Condenser mic | Short boom | Optional for smaller rooms. |
| 10 | Rack tom | Tom mic | Clip/short boom | If kit has rack tom. |
| 11 | Floor tom | Tom mic | Clip/short boom | If kit has floor tom. |
| 12 | Overhead L | Condenser mic | Tall boom | Stereo overhead pair. |
| 13 | Overhead R | Condenser mic | Tall boom | Stereo overhead pair. |

# Sax reverb setup

Preferred: two separate sax channels at FOH - one dry/normal and one wet/reverb. The wet channel should use a large hall or plate-style reverb and be blended tastefully during soundcheck.

Fallback: if two physical sax mics are not possible, split one sax mic into two console channels and process one as dry and one as wet/reverb.

# Monitor requirements

| Mix | Position | Main needs |
| --- | --- | --- |
| M1 | Hybrid guitar | Guitar amp, bass amp, sax, keys, some kick/snare. |
| M2 | Tenor sax | Sax, guitar, keys, some drums. |
| M3 | Keyboard | Keys, guitar, sax, kick/snare. |
| M4 | Drums | Hybrid guitar bass side, keys, sax, enough guitar side for pitch. |

Minimum workable setup: 2 monitor mixes. Preferred: 4 independent mixes.

# Backline request

1 clean guitar amp suitable for jazz/clean electric guitar tone.

1 bass amp suitable for the low/bass side of the hybrid guitar.

Standard drum kit: kick, snare stand, hi-hat stand, rack/floor toms, cymbal stands and drum throne. Drummer usually provides cymbals, snare and breakables unless agreed otherwise.

Keyboard stand, sustain pedal and mains power near keyboard position. Keyboard player to bring keyboard unless venue backline is agreed in advance.

At least 2 tall boom stands, 3 short boom stands, 2 DI boxes for keyboard and one spare DI if possible.

4 wedge monitors preferred.

# Power

Two clean power points near guitar position for pedals/amps if required.

Two clean power points near keyboard position.

Power should be safely taped/secured and not run through audience walkways.

# Soundcheck and notes for FOH

Ideal soundcheck: 45 minutes after full line check. Minimum line check: 20 minutes.

Keep the hybrid guitar guitar-side and bass-side clear and separated. The bass side should carry weight but not overpower the acoustic drums.

Dry sax should stay present and natural. Wet sax/reverb channel should feel spacious but not wash out the melody.

Keyboard can be stereo where the room and desk allow; mono is acceptable for small venues.

No backing tracks or click are required unless confirmed separately.

# Advance checklist

Confirm whether venue or artist provides guitar amp and bass amp.

Confirm whether the sax wet/reverb channel will be created with two mics or one mic split to two channels.

Confirm drum kit/backline details and what the drummer brings.

Confirm keyboard provision and number of DI boxes.

Confirm monitor count and independent mixes.

Send stage dimensions and venue curfew/load-in information.